YEDITEPE UNIVERSITY

DEPARTMENT OF COMPUTER ENGINEERING

INTRODUCTION TO DIGITAL SYSTEMS LABORATORY- LAB #5

Spring 2022

Objective:

Sequential Circuit Design and Implementation at Register-Transfer Level (RTL).

General Information:

In this lab, you will design and implement sequential circuits which are described at register-transfer level. You are going to use Verilog hardware description language (HDL) both to model your design and implement it.

Lab Equipment:

Xilinx ISE Design Suite, Digilent Basys2 or Nexys2 or Nexys4 board.

Design #1: Seven-Segment Banner

Figure below shows the block diagram of seven-segment banner design. Notice that there are clk (clock) and rst (reset) signals in the design. There are also a read-only memory (ROM) and a state machine within the design.

Procedure:

- Create a new folder named lab_5 under Desktop folder.

- Invoke Xilinx ISE Design Suite.

- Create a new project.

  - Click on New Project… under File menu.

  - Set the project name, location, and working directory as follows:

    - Name:

rotating_banner

    - Location:

C:\Users\<USER>\Desktop\lab_5\rotating_banner

    - Working Directory:

C:\Users\<USER>\Desktop\lab_5\rotating_banner

  - In Project Settings window, set Family, Device, and Package as follows:

    - If you got Basys2 Board:

    - If you got Nexys2 Board:

    - If you got Nexys4 Board:

  - Don’t change anything in Project Settings window. Just click on Next.

  - Check Project Summary window to make sure that your project settings are correct. After that, click on Finish.

- Create a new source file to describe the top design.

  - Click on New Source… under Project menu.

  - Select Verilog Module and set File name as top in Select Source Type window.

  - Don’t change anything in Define Module window. Just click on Next.

  - Check Summary window to make sure that your module settings are correct. After that, click on Finish.

- Complete the module as in the following figure and then click on  button to save your design.

- Download slowdown_unit module from COADSYS as .v files and add them into the design.

  - Click on Add Copy of Source… under Project menu.

  - Select the downloaded Verilog files and click on Open and then click on OK.

- Create a new source file to describe the character_rom design.

  - Click on New Source… under Project menu.

  - Select IP (CORE Generator & Architecture Wizard) and set File name as character_rom in Select Source Type window and then click on Next.

  - Select Block Memory Generator under RAMs & ROMs of Memories & Storage Elements. Click on Next and then Finish.

  - Skip the first page by clicking on Next.

  - Select Single Port ROM as Memory Type on the second page and click on Next.

  - Set Read Width as 8 and Read Depth as 30 on the third page and then click on Next.

  - Check the Loan Init File option on the fourth page and set the name of Coe File as character_rom.coe (Download character_rom.coe file from COADSYS).Then click on Generate.

  - After character_rom file is generated, click on character_rom module  and then View HDL Instantiation Template under Core Generator  window.

- Create a new source file to describe the user constraints of top design.

  - Click on New Source… under Project menu.

  - Select Implementation Constraints File and set Filename as top in Select Source Type window. After that, click on Next.

  - Click on Finish to add the file into your project.

- Add the following clock constraint to the top.ucf file and use the below figures to describe the user constraints of your top design.

NET "clk" LOC = "B8"; # (FOR BASYS2 AND NEXYS2)

NET "clk" LOC = "E3" | IOSTANDARD = "LVCMOS33"; # (FOR NEXYS4)

Basys2 Circuit Figure

Nexys2 Circuit Figure

Nexys4 Circuit Figure

- While on Implementation view, click on Implementation to start implementing the design.

- After Implementation is completed click on Generate Programming File to generate “.bit” file.

- After connecting the FPGA board to the computer and it is powered on, invoke Digilent Adept.

- After invoking Adept Select “top.bit” from the project folder using browse and then press Program button.

- After Program is successfully loaded to the FPGA board, check whether your design works properly.

Design #2: Rotating Seven-Segment Banner

Modify the existing design to implement a rotating seven-segment banner. Display 7tEPE on seven-segment displays and add a switch to control the direction of state transitions.

Since we are dealing with a sequential circuit, we need to input a clock signal to our design. In our case, we will use a

- 50 MHz (20 ns) clock signal (for Basys2 and Nexys2)

- 100 MHz (10 ns) clock signal (for Nexys4)

while implementing our design.

| CHARACTER MAP of character_rom module |  |
| --- | --- |
| ADDRESS; DATA. 0; 0. 1; 1. 2; 2. 3; 3. 4; 4. 5; 5. 6; 6. 7; 7. 8; 8. 9; 9. 10; A. 11; b. 12; C. 13; d. 14; E. | ADDRESS; DATA. 15; F. 16; g. 17; h. 18; ı. 19; j. 20; L. 21; n. 22; o. 23; P. 24; r. 25; S. 26; t. 27; u. 28; y. 29; Z. |